# µFalcon-RX

Compact Edge xHaul PTP Switch & Clock

| Product Overview The µFalcon-RX is an essential element in delivering on the promise of 5G. Through high capacity, low latency transport and high precision synchronization, the µFalcon-RX enables 5G RAN operation at its optimal capacity, on both telecom and enterprise environments. The µFalcon-RX combines the best of breed of the Transport and Timing worlds, in a single powerful package. Both elements are critical for 5G fronthaul deployments, along with support for the latest O-RAN architectures and recommendations. The µFalcon-RX’s Timing toolbox includes a complete set of capabilities, including GNSS based PTP GM/BC/TC/OC (sub nanosecond accuracy, Class C/D performance), along with robust SyncE and other sync interfaces. The system’s timing is orchestrated via Fibrolan’s user friendly SyncCenter. The µFalcon-RX series is equipped with a total of 6xSFP+ and 2xSFP28 ports, with configurable port setup for up to 110Gbps Full Duplex. All ports can operate at full wire speed, at any packet size (including Jumbo frames). The µFalcon-RX offers advanced Quality of Service (QoS) features including classification and mapping based on layer 1 through layer 4 attributes, port and queue policing and shaping, with highly flexible scheduling schemes. Support for Time Sensitive Networks (TSN) makes the µFalcon-RX an even greater fit for fronthaul, as well as industrial environments (Industry 4.0, IIoT), where 5G infrastructure serves the automation of factories. # The µFalcon-RX fully supports O-RAN LLS-C1 through C4 configuration including the | LTE/5G xHaul Transport and Timing switch Integrated PTP Grandmaster Compatible with O-RAN architectures High capacity, low latency Extensive Sync and Timing with SyncE and PTP (PTRC/GM, BC, TC) Sub nanosecond timestamping, Class C/D performance Time Sensitive Networking support Based on 4th generation Falcon architecture Advanced QoS and service level traffic management Advanced OAM and management capabilities Multiple protection mechanisms for link, path, and ring service resilience Compact, low power forwarding of C/U-plane eCPRI packets and S/M planes for management and synchronization. Multiple protection schemes help cover any deployment topology (linear, star, ring) and deliver the reliability required for critical infrastructure such as 5G. In addition, L3 forwarding is supported (static; dynamic routing in future SW releases). The system implements effective OAM tools, for monitoring, alarming, analysis and troubleshooting of the system and the forwarding plane as well as the synchronization plane. The µFalcon-RX is housed in a highly compact, half-19”, 1U chassis (150mm deep only) and has an integrated internal, dual feed DC power supply. |
| --- | --- |

| Technical Specifications |  |
| --- | --- |
| Interfaces & Indicators |  |
| Ethernet: 6 x 1/2.5/10G (SFP+) 2 x 1/2.5/10/25G (SFP28) - Supported SFP/SFP+: MM, SM, SFS, xWDM, Copper Sync & Timing: All optical ports support PTP, SyncE and NTP GNSS antenna in (SMA, active, 5VDC) 2 x external 1PPS/10MHz (SMA) | Management (OOB): 1 x 10/100/1000BaseT (RJ45) 1 x USB (console) LEDs Link/Activity (per port) Sync GNSS CPU Power |
| Architecture & Forwarding |  |
| Hybrid (ASIC-FPGA) HW architecture L2 forwarding (802.1D MAC bridging) Flow-based forwarding Performance: wire-speed, on all ports, all frame sizes Switching fabric: 220Gbps (110Gbps/FDX), non-blocking MTU: 10K bytes MAC table: 32K addresses VLANs: 4K concurrent Provider bridging: 802.1ad (Q-in-Q) | Private VLANS L1-L4 ACLs Multicast: IGMPv3 snooping MLD snooping Up to 8K MC groups Layer 3: Static routes IPv4/IPv6 DHCP (client, server, relay) Time Sensitive Networking |
| Management |  |
| Interfaces: CLI: Console, Telnet, SSH SNMP: v1/v2c/v3, extensive MIBs Web: HTTP/HTTPS Management VLAN IPv6 management Authentication: RADIUS, TACACS+ Multiple local users User access levels (15) Management ACLs 802.1x (port/MAC based) DHCP client, relay, server, snooping Link discovery: LLDP, LLDP-MED, CDP ware | CPU sub-system: Dual-core, @1GHz, ARM 1GB DDR, 4GB flash memory Operations: Remote System Update (TFTP or Web) Configuration upload/download (TFTP or Web) Text based config files Alarms: SNMP traps Syslog (internal and remote server) CLI events Remote temperature reading & alarm Per port and queue detailed statistics RMON |
| OAM & Diagnostics |  |
| IEEE802.3ah Link OAM IEEE802.1ag CFM Ping (IPv4/v6) Traceroute (Ipv4/v6) | Throughput metering SFP diagnostics (SFF-8472) Traffic mirroring and remote mirroring sFlow |
| Protection |  |
| Link aggregation: static or LACP Loop protection | Linear protection: G.8031 Ring protection: G.8032v2 Spanning tree: STP, RSTP, MSTP |

| PTP/IEEE1588v2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Functions: Grandmaster (PRTC-A) Boundary Clock (Class C/D) Ordinary Clock (M/S) Transparent Clock (Class C/D) Profiles supported: Telecom Frequency (G.8265.1) Telecom Phase (G.8275.1, G.8275.2) Default (IEEE1588v2) 802.1AS Custom |  |  |  | Modes supported: 1 and 2 step L2 Multicast L3/UDP Unicast/Multicast Mixed transport modes E2E and P2P delay VLAN tagging Slave capacity: Up to 256 Unicast @ full packet rate Support for max packet rates for: Announce, Sync and Del.Req/Del.Resp messages HW timestamping: <1nsec resolution |  |  |  |  |
| Other Timing Services/Features |  |  |  |  |  |  |  |  |
| Synchronous Ethernet (SyncE): G.8261, G.8262 ESMC (G.8264) GNSS: 32 channels Multi-constellation : GPS, GLONASS, Galileo, Beidou, QZSS Support O-RAN configurations LLS-C1/C2/C3/C4 |  |  |  | SyncCenter NTP Client Server External Sync: Input/output 1PPS/10MHz Local clock: Built-in Stratum 3E clock |  |  |  |  |
| Holdover Performance |  |  |  |  |  |  |  |  |
|  | - Oscillator \ T.Err | - 200nsec | - 400nsec |  | - 1usec | - 1.5usec | - 5usec | - - |
|  | - HQ-OCXO | - 1.5 hours | - 3 hours |  | - 5 hours | - 8 hours | - 22 hours |  |
| Note: the above are typical values, assuming the system was locked to GNSS for at least 72 hours, at a constant temperature and the system was powered for at least 30 days. |  |  |  |  |  |  |  |  |
| Quality of Service |  |  |  |  |  |  |  |  |
| Classification based on L1-L4 information Ingress policing per port/queue Hierarchical shaping and scheduling Priority based flow control (802.1Qbb) Scheduling: Strict, DWRR, hybrid |  |  |  | 4 drop precedence levels WRED and tail drop for CA P-bit and DSCP remarking Storm control: UC, MC, BC QoS Control Lists |  |  |  |  |
| Power & Environmental |  |  |  |  |  |  |  |  |
| Power Supply: Internal power supply: 20-60VDC, dual feed AC adapter option (FPA40) Power consumption: Maximum: <30W; typical: <25W |  |  |  | Operating temperature: Standard: -10°C ÷ +50°C (14°F ÷ 122°F) Extended: -40°C ÷ +65°C (-40°F ÷ 149°F) (optional) Storage temperature: -40°C ÷ +80°C (-40°F ÷ 176°F) Humidity: 10-90%, non-condensing |  |  |  |  |
| Physical |  |  |  |  |  |  |  |  |
| Dimensions (HxWxD): 44x221x150mm (1.73x8.70x5.90 inch) Mounting: Desktop, Rack, Wall Weight: ~0.8Kg (1.76 lb) |  |  |  | Accessories: Power cable USB cable (console) Rack mounting kit (optional) |  |  |  |  |
| Regulatory & Compliance |  |  |  |  |  |  |  |  |
| Safety: IEC EN60950-1 CE RoHS |  |  |  | EMC: FCC CFR 47 part 15, subpart B, Class A EN 300 386 V1.3.3: 05 |  |  |  |  |

Typical Application: Private LTE/5G/CBRS

Ordering Information

| Model | P/N | Description |
| --- | --- | --- |
| µFalcon-RX/206/G/D | 7162 | Timing Aware Compact Edge xHaul Switch, 6x10G (SFP+), 2x25G (SFP28) ports, Advanced Timing spec w/ GNSS Rx, internal DC (20-60VDC) dual feed power supply, and TNC to SMA GNSS cable adapter |
| FPA40 | 7108 | AC (100-240V) to DC (48V) power adapter, 40W |

Specifications are subject to change w/o prior notice

| Intl. Headquarters Fibrolan Ltd. Tel: +972-4-959-1717 Fax: +972-4-959-1718 info@fibrolan.com www.fibrolan.com | North America Fibrolan Inc. Tel: +1-201-843-1626 Fax: +1-201-843-1628 info.us@fibrolan.com www.fibrolan.com | Central-Eastern Europe Fibrolan CEE GmbH. Tel: +43 2253 21188-0 Fax: +43 2253 21188-99 Info.cee@fibrolan.com www.fibrolan.com |
| --- | --- | --- |

Revision: uFalcon-RX_DS_2026-09-27_v1-9d

© Fibrolan 2026. All Rights Reserved

It’s About Time

‹#›